ITU - Telecommunication Standardization Sector	Temporary Document XXX

STUDY GROUP 16

Geneva, 14 – 25 September 1998

Question(s):	13/16

SOURCE*:	Rapporteur for Q.13/16 (Dale SKRAN)

TITLE:	Draft H.225.0 Annex G – Communication Between Administrative Domains

Summary:

H.225.0 Annex G describes methods to allow address resolution between administrative domains in H.323 systems for the purpose of completing calls between the administrative domains. Annex G describes the system elements involved in this communication, the procedures for information exchange, and the protocol for information exchange. Use of the proposed scheme for communication between gatekeepers within an administrative domain is for further study.

*Contact:	Glen Freundlich, editor H.225.0

Lucent Technologies			tel.: +1 303 538 2899

11900 N. Pecos				fax: +1 303 538 3907

Westminster, Colorado 80234 USA	email: ggf@lucent.com

# 1	ANNEX G - Communication Between Administrative Domains

## 2 Scope

This annex describes methods to allow address resolution between administrative domains in H.323 systems for the purpose of completing calls between the administrative domains. An administrative domain exposes itself to other administrative domains through a type of edge element known as a border element. Annex G does not require an administrative domain to reveal details about its organization or architecture. Annex G does not mandate a specific system architecture within an administrative domain. Furthermore, Annex G supports the use of any call model (gatekeeper routed versus direct endpoint).

## 3 Definitions

| Administrative domain | An administrative domain is a collection of equipment administered by one administrative entity. An administrative domain can consist of one or more gatekeepers (that is, one or more zones). An administrative domain also contains all the components required for offering services beyond a simple H.323 call. |
| --- | --- |
| Aggregation Point | A border element that may be able to resolve a set of addresses including the addresses which are not in its administrative domain. |
| Back-End Services | Back-End Services are functions such as user authentication or authorization, accounting, billing, rating/tariffing, etc. Back-end services and the protocol to exchange information with back-end services (if different than that in this Annex) are not in the scope of this Annex. |
| Border element | The border element is a functional element which supports public access into an administrative domain for the purposes of call completion or any other services that involve multimedia communication with other elements within the administrative domain. The border element controls the external view of the administrative domain. A border element communicates with other border elements using the protocol specified in this Annex. In addition, a border element may, depending on implementation, communicate with other entities within its administrative domain via RAS (for example, LRQ/LCF with gatekeepers in the administrative domain) and H.225.0 and H.245 (for example, if the border element routes call signaling). This element may exist in combination with other H.323 elements, for example a combination of border element, gatekeeper, and gateway. An administrative domain may contain any number of border elements. |
| Clearing House | A service (possibly in the form of a border element) which can provide resolution for all addresses. |

## 4 Description

The information exchanged through the Annex G protocol supports completion of calls between administrative domains. This information may include address resolution, authentication, authorization, call details, and so on.

Figure 1 indicates a number of references points representing signaling among various elements. In this figure, the administrative domains are part of a global packet network without edges. Note that this figure is not an explicit definition of an H.323 system architecture, but is meant to illustrate signaling reference points.

Figure 1 - System Reference Points

The figure indicates the following reference points:

A – between border elements

B – between border element and gatekeepers

C – between gatekeepers

D – between H.323 elements and back end services (not in the scope of this Annex)

Reference point A is the focus of Annex G. Use of the protocol described in Annex G for communication between gatekeepers within an administrative domain is for further study. Reference point B is considered for further study since it is currently assumed that the border element will be colocated with some other H.323 element.

### 5 Requirements

[ editor: Annex G will be created by first addressing requirements that have been determined to be of higher priority. Requirements determined to be of lower priority may be addressed in the first version of Annex G if time exists in the approval schedule. These requirements were set at the Yokosuka meeting and will be removed from this Annex before it is brought for decision .]

Higher priority requirements:

- An originating endpoint controlled by one gatekeeper, or the gatekeeper itself, can obtain the address of a destination entity controlled by terminating GK, or an address to the destination entity (for example, the destination’s gatekeeper if gatekeeper routing is used). Effort should be made to use existing technologies where appropriate.

- Gatekeepers (border elements) can exchange some notion of policies that will allow a gatekeeper to select a destination based on some level of service, capacity constraints, or other inter-domain relationships. (This is a reflection of some form of application-level “QoS”.)

- Border elements can mutually authenticate. An element may want to authenticate another element before exchanging zone information.

- Border elements can mutually authorize so that an endpoint in one domain can access a service in another domain.

- Information, such as tokens, can pass end-to-end when intermediate points do not interpret the information.

- Border elements can support non-repudiation.

- Annex G should not require modification of existing protocols, so that those protocols maintain backward compatibility with V1 or V2 equipment. However, it is reasonable to assume that these protocols may be extended to provide added functionality. Any changes to the existing protocols should be reflected where they are currently defined.

Lower priority requirements:

- When a user attempts to gain service from a “foreign” domain (not the user’s home domain), that domain can provide service somehow related to the user’s home domain definition and services. In addition, the foreign domain can update the user’s home domain so that calls can be redirected to the foreign domain. Existing models (e.g., GSM, AMPS) should be examined to see if those models, or parts, can be used. (This is the “roaming” requirement from Yokosuka APC-1341.)

- Information can be exchanged between administrative domains such that either domain can terminate (drop) a call, or provide some other feature, based on some policy. For example, this policy might be to terminate a call when the caller’s account (e.g., phone card) has run out of money.

- A gatekeeper can authenticate an entity known to some other gatekeeper in some other domain.

Goals:

- Try to define a protocol that runs in the background as much as possible, not necessarily for every call. Call signaling should not be gated by information exchange between domains prior to each call. However, this possibility should be allowed if implementations desire.

- The architecture should be highly scalable, but there is probably no need to quantify the scale.

- Topologies behind the border elements need not be known outside that domain. Information can be exchanged between domains without revealing information about zone topology.

- Utilize H.235.

## 6 References

## 7 System Models

Annex G does not mandate a specific system architecture among administrative domains or within an administrative domain. The following subsections will provide some sample architectures, but these are to be viewed as illustrative rather than exhaustive.

In general, an administrative domain is viewed as consisting of any number of zones and any number of border elements. Remember that a border element is a functional element that may exist together with any other H.323 element. Figure 2 shows some examples of border element implementations in combination with other elements.

Figure 2 - Border Element Placement Examples

The relationship among administrative domains may be any of a variety of organizations. The following subsections indicate example relationships.

### 8 Hierarchical

Figure 3 shows a simple hierarchical arrangement among administrative domains. In such an arrangement, a border element in an administrative domain would consult a border element in an administrative domain higher in the hierarchy to resolve an address.

Figure 3 - Sample Hierarchical Organization

### 9 Distributed or Full Mesh

An entirely distributed or full mesh model is possible, as shown in Figure 4. In this example, a border element in each administrative domain communicates with border elements in the other known administrative domains.

Figure 4 - Sample Distributed Organization

### 10 Clearing House

An example of a clearing house arrangement is shown in Figure 5. In this arrangement, each administrative domain consults the clearing house to resolve addresses.

Figure 5 - Sample Clearing House Organization

### 11 Aggregation Point

Figure 6 shows an example of an aggregation point. In this example, administrative domain B is an aggregation point that can provide address resolution for both itself and administrative domain C. Administrative domain B may cache C’s address information or may serve as a tandem between A and C.

Figure 6 - Aggregation Point Example

### 12 Overlapping Administrative Domains

OPEN: The possibility of zones or users being in multiple administrative domains was discussed at the September 1998 SG16 meeting. Text is needed to address this possibility.

## 13 Addressing Conventions

In order to provide interoperability between domains it is important that the addressing formats sent in H.323 messages are understood by the receiving system. A border element shall support both the email-id and partyNumber types of AliasAddress. Note that this requirement implies support of H.225.0 V2 or later. When communicating with other border elements, only the email-id and partyNumber types of AliasAddress should be used in the destinationAddress field of an LRQ or SETUP message unless there has been prior agreement between the Administrative Domains concerned.  For example, if a group of Administrative domains have agreed on the interpretation of private local numbers then these numbers may be used in messages between them.

## 14 Operation

### 15 Address Templates

An address template defines a set of AliasAddress identifiers, and the protocol to be used in reaching addresses in that set. Template information may allow the aggregation of addressing information if the addressing scheme is arranged in some hierarchical or routable manner (for example, a given zone might handle 1303538*, meaning all telephone numbers that begin with 1303538).

Examples include:

“For +1 555 123 4567	send Address Query to 1.2.3.4”

“For +1 555 987 *	send Address Query to 5.6.7.8”

“For +1 555 987 6543	send Setup message to 5.6.7.9”

“For *@example.org	send Address Query to 123.4.5.6”

“For +1 *		send Address Query to 8.8.8.8”

“For private 31*		send Address Query to 11.12.13.14”

“For +44 171 112*”	doesn’t exist”

A border element obtains templates in these ways:

- static configuration

- receiving zone descriptors from other border elements in response general requests

- receiving responses to specific queries

- input from foreign databases

#### 16 Static Configuration

A border element will be configured with templates for all the zones for which it is responsible. These templates may be explicitly provisioned in the border element, or these templates may be formed by summarizing information obtained from gatekeepers within its domain. The border element may make this information available to other border elements via responses to requests. An administrative domain may choose the level of detail to be provided by its border elements.  Examples include:

- A border element that wishes to hide internal structure might provide one descriptor (with an indication to send an Address Query message) which describes its whole zone and refers to a gatekeeper which will handle all incoming calls.

- A border element which does not care about revealing internal structure might provide a set of templates, each describing the gatekeeper for a zone within the domain.

- A border element which is on a firewall (or one using the gatekeeper routed model) might provide a template for the whole zone with an indication to send a Setup message.

- A border element with holes in its domain (because numbers have been moved to another administrative domain) provides templates marked “Send Address Query” which indicate the border element which should be used to contact the other administrative domain.

- A clearing house border element (such as one which has a complete copy of +44) might hold a template marked “Send Address Query” for each administrative domain within +44.

Border elements need not keep a copy of the whole database.  If a border element does not hold a copy of the whole database then it should contain statically configured “Send Address Query” templates indicating a clearing house border element which will be used to resolve other queries.  These templates are known as last-resort templates.

#### 17 Receiving Zone Descriptors

A border element may request the statically configured templates from another border element (apart from the last-resort templates).  The response to the request is decided by the border element from which the templates are being requested.

To initiate a complete transfer the requesting border element opens a reliable connection (e.g., a TCP connection) to the owning border element and sends a Zone Request message specifying the zones it wishes to transfer. If the owning border element is able to transfer the zone descriptors it responds with a ZoneRequestConfirmation specifying all the templates in the zone.

The requesting border element may cache a copy of a templates transferred in this way until the template’s lifetime expires, at which point the border element shall delete its copy of the template. If the owning border element changes its statically configured templates before their lifetime has expired then it shall send a ZoneUpdate message to those border elements of which it is aware. A border element in receipt of a ZoneUpdate message shall either delete all indicated templates from its cache, or shall request copies of the indicated zones from the owner.

A border element may send a Zone Update message to other known border elements, or the border element may multicast a Zone Update message. If a Zone Update message is multicast, the border element should consider the scope of the multicast. The Zone Update message can contain the descriptors that have changed. Alternatively, the Zone Update message may indicate only the identification of the zones that changed, allowing the recipient to query for the new information. If a large number of descriptors have changed, the information should be sent in multiple Zone Update messages so that a particular Zone Update message does not exceed the maximum transport packet size.

#### 18 Receiving Responses to Specific Queries

A border element may send an AddressQuery to another border element asking for the resolution of a specific query.  The Address Query is sent over unreliable transport (e.g., UDP), and contains questions about one or more Templates.

A border element in receipt of a specific query searches its database and responds with the most specific template for the destination. If the desination border element is actually responsible for the alias address specified will usually respond with a template indicating that either an Address Query or Setup message should be sent. If the destination border element is a clearing house, it will normally respond with a template indicating that the Address Query message should be sent.

The destination border element may also add templates to the response which it believes will be useful in the future. The addition of these templates should not make the response so large that the transport network will need to fragment it.  (eg 576 bytes for IPv4 or 1200 bytes for Ipv6).

For example, a border element which is tightly coupled with a firewall may provide two templates in its response to Address Query messages which it can answer: one template with a short lifetime (of a few minutes or seconds) specifying the location to which a Setup message should be sent, and additional templates specifying that Address Query messages should be sent to the border element for other AliasAddresses within the administrative domain.

A border element may cache answers to specific queries until their lifetime expires.

#### 19 Input from Foreign Databases

Ownership of Email-ID style addresses is defined by the DNS system. Thus, in the absence of any better information, a border element may do a DNS SRV record lookup on the part of the email-ID to the right of the ‘@’ sign. The response to this lookup should be used to synthesise a “Send Address Query” template which can be used during the resolution process. Templates synthesised from DNS requests should not be cached for longer than the lifetime provided in the DNS response.

### 20 Finding a Border Element or Set of Border Elements

#### 21 Administered

A border element may have an administered set of other border elements which it may contact for address resolution. This administered set may be defined through a set of bilateral agreements between the administrative domains and other administrative domains.The  administrative domains  may optionally utilize the service of a clearing house.

#### 22 Using DNS

OPEN: text is needed here

#### 23 Other Methods

The use of other methods to locate another border element are for further study.

### 24 Resolution Procedures

#### 25 Resolution Procedure Within Administrative Domain

When a border element is asked to resolve an AliasAddress (e.g., by a co-located gateway or gatekeeper), it searches through the templates in its cache and finds those which match.

If more than one template matches, they are first sorted by wildcard length (more specific templates are better). They are then sorted by the type of protocol specified (“Send Setup” is better than “Address Query”).

If the first template is marked as “Send Setup message”, then the resolution process has found an answer. The address to which to send the Setup message is returned to the entity which sent the request.

If the first template is marked as “Send Address Query”, then the border element sends an Address Query message with a specific destination address to the address specified in the template.  When it gets an answer from the border element it may store that in its cache and return to the requester the address to which to send the Setup message.

#### 26 Resolution Procedure Between Administrative Domains

When a border element receives an Address Query it searches through the templates in its cache and finds those which match the addresses in the query.

If more than one template matches, they are first sorted by wildcard length (more specific templates are better). They are then sorted by the message type specified (“Send Setup” is better than “Address Query”).  In each case all templates other than the most specific match are discarded.

If the matched templates are marked as “Send Address Query” then the border element may choose to send the address query itself, or may choose to return the templates as they are.

The border element then returns an Address Query Response message containing the templates which it has found (these will have the same address and message type fields) and any other templates which it considers will be useful.

## 27 Protocol

### 28 Message Definitions

#### 29 Zone Descriptor

The Zone Descriptor is not a message, but is rather the primary message element used to describe a zone. The Zone Descriptor provides an indication of the addresses that can be resolved in the zone.

The Zone Descriptor contains the following information:

| Field | Description |
| --- | --- |
| Zone ID | A globally unique ID to identify this zone from among many possible zones |
| Gatekeeper ID | A text identifier that indicates the owner of the zone (i.e., the gatekeeper that created this message) |
| Templates | A set of templates which define the addresses this zone can resolve. |

#### 30 Template

The Template describes a set of (where set size can be 1) alias addresses. The Template is not a message, but is an element used as a building block for other elements.

| Field | Description |
| --- | --- |
| Pattern | A description of a set of alias addresses. The addresses can represent telephone numbers (E.164) or email addresses. Addresses can be represented with meta-characters or some other hierarchical set definition to represent expansion of the string. For example, the pattern “+1 303*” could represent any number in the Denver area code. [editor: the definition of pattern will be expanded to provide detailed instructions on the use of meta-characters] |
| Transport Address | The address to which to send the message indicated in “Message Type” |
| Message Type | Indicates the type of message to send when attempting to resolve a specific address within this template. Possibilities are Address Query or Setup. |
| Pattern type | Describes the endpoints addressed by “Pattern” above. |
| TimeToLive | Indicates the time up to which this pattern is valid |
| Transport QoS | Describes the QoS capabilities of the pattern |
| Authentication | A set of authentication mechanisms supported by entities represented by this template. |

#### 31 Zone Request

The zone request message allows an entity to query a border element regarding the addresses that the border element’s domain can resolve.

| Field | Description |
| --- | --- |
| Sequence number | Monotonically increasing message sequence number to be used in any reply to this message |
| Version | Protocol version in use by the sender of this message |
| Zone ID | Allows requests for a specific zone |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 32 Zone Request Confirmation

The zone confirmation message is a border element’s response to a zone request, when the border element can interpret the request and implementation rules allow information exchange.

| Field | Description |
| --- | --- |
| Sequence number | Sequence number from the zone request |
| Version | Protocol version in use by the sender of this message |
| Zone descriptor | The zone descriptor described above |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 33 Zone Request Rejection

A border element can reject a zone request for a variety of reasons.

| Field | Description |
| --- | --- |
| Sequence number | Sequence number from the zone request |
| Version | Protocol version in use by the sender of this message |
| Reject reason | Undefined reason, invalid zone request seqnum, resource unavailable, security denial, zone ID error |
| Zone ID | Identifies the specific zone for this response |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 34 Zone ID Request

The Zone ID Request allows an entity to query a border element for a list of zones within the border element’s administrative domain.

| Field | Description |
| --- | --- |
| Sequence number | Monotonically increasing message sequence number to be used in any reply to this message |
| Version | Protocol version in use by the sender of this message |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 35 Zone ID Request Confirmation

A zone ID confirmation message is a border element’s response to the Zone ID Request message.

| Field | Description |
| --- | --- |
| Sequence number | Sequence number from Zone ID Request |
| Version | Protocol version in use by the sender of this message |
| Zone IDs | A list of zone identifiers that the border element wishes to reveal outside the administrative domain |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 36 Zone ID Request Rejection

A border element can reject a zone ID request for a variety of reasons.

| Field | Description |
| --- | --- |
| Sequence number | Sequence number from Zone ID Request |
| Version | Protocol version in use by the sender of this message |
| Reject reason | Reasons for rejecting the request |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 37 Zone Update

The Zone Update message is a border element’s notification that address information has changed. A border element may also send the Zone Update message during initialization. A border element in receipt of the Zone Update may request information from the element identified in the Zone Update.

| Field | Description |
| --- | --- |
| Sequence Number | Monotonically increasing message sequence number |
| Version | Protocol version in use by the sender of this message |
| Transport address | An element in receipt of the Zone Update may send a request to this address. |
| Zone IDs | A set of globally unique IDs to identify the zone(s) that have changed |
| Descriptors | The set of descriptors that have changed |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 38 Address Query

A border element can send an Address Query message to another border element to ask for resolution of a specific alias address.

| Field | Description |
| --- | --- |
| Sequence Number | Monotonically increasing message sequence number to be used in any reply to this message |
| Version | Protocol version in use by the sender of this message |
| destinationInfo | The adress to be resolved |
| Reply address | The address to which to send the reply to this message |
| tokens |  |
| cryptoTokens | Encrypted tokens |
| Non standard | Non standard information |
| integrityCheckValue | … to support some integrity scheme |

#### 39 Address Query Confirmation

A border element returns address query information in the Address Query Confirmation message.

| Field | Description |
| --- | --- |
| Sequence number | Sequence number from the Address Query |
| Version | Protocol version in user by the sender of this message |
| descriptor | A descriptor which match the attributes of the query |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

#### 40 Address Query Rejection

A border element can reject an Address Query for a variety of reasons.

| Field | Description |
| --- | --- |
| Sequence number | Sequence number from the Address Query |
| Version | Protocol version in use by the sender of this message |
| Reject reason | Reasons for rejecting the request |
| Tokens |  |
| Crypto tokens | Encrypted tokens |
| Non standard | Non standard information |
| Integrity check value | … to support some integrity scheme |

### 41 Message Syntax

AliasAddress ::= CHOICE

{

email-ID	IA5String (SIZE(1..512)),	-- rfc822-compliant email address

partyNumber	PartyNumber,

...

}

PartyNumber ::= CHOICE

{

publicNumber PublicPartyNumber,

-- the numbering plan is according to

-- Recommendations E.163 and E.164.

dataPartyNumber NumberDigits,

-- not used, value reserved.

telexPartyNumber NumberDigits,

-- not used, value reserved.

privateNumber PrivatePartyNumber,

nationalStandardPartyNumber NumberDigits,

-- not used, value reserved.

...

}

PublicPartyNumber ::= SEQUENCE

{

publicTypeOfNumber	PublicTypeOfNumber,

publicNumberDigits	NumberDigits

}

PrivatePartyNumber ::= SEQUENCE

{

privateTypeOfNumber	PrivateTypeOfNumber,

privateNumberDigits	NumberDigits

}

NumberDigits ::= IA5String (SIZE (1..128)) (FROM (“0123456789#*,”))

PublicTypeOfNumber ::= CHOICE

{

unknown			NULL,

-- if used number digits carry prefix indicating type

-- of number according to national recommendations.

internationalNumber	NULL,

nationalNumber		NULL,

networkSpecificNumber	NULL,

-- not used, value reserved

subscriberNumber	NULL,

abbreviatedNumber	NULL,

-- valid only for called party number at the outgoing

-- access, network substitutes appropriate number.

...

}

PrivateTypeOfNumber ::= CHOICE

{

unknown			NULL,

level2RegionalNumber	NULL,

level1RegionalNumber	NULL,

pISNSpecificNumber	NULL,

localNumber		NULL,

abbreviatedNumber	NULL,

...

}

GloballyUniqueID	::=	OCTET STRING (SIZE(16))

SequenceNumber		::=	INTEGER (1..65535)

GatekeeperIdentifier	::=	BMPString (SIZE(1..128))

ProtocolIdentifier	::=	OBJECT IDENTIFIER

-- shall be set to

-- {itu-t (0) recommendation (0) ???}

TransportAddress ::= CHOICE

{

ipAddress	SEQUENCE

{

ip		OCTET STRING (SIZE(4)),

port		INTEGER(0..65535)

},

ipSourceRoute	SEQUENCE

{

ip		OCTET STRING (SIZE(4)),

port		INTEGER(0..65535),

route		SEQUENCE OF OCTET STRING(SIZE(4)),

routing 	CHOICE

{

strict	NULL,

loose	NULL,

...

},

...

},

ipxAddress	SEQUENCE

{

node	OCTET STRING (SIZE(6)),

netnum	OCTET STRING (SIZE(4)),

port	OCTET STRING (SIZE(2))

},

ip6Address	SEQUENCE

{

ip	OCTET STRING (SIZE(16)),

port	INTEGER(0..65535),

...

},

netBios			OCTET STRING (SIZE(16)),

nsap			OCTET STRING (SIZE(1..20)),

nonStandardAddress	NonStandardParameter,

...

}

EndpointType ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

vendor			VendorIdentifier,

gatekeeper		GatekeeperInfo OPTIONAL,

gateway			GatewayInfo OPTIONAL,

mcu			McuInfo OPTIONAL,	-- mc must be set as well

terminal		TerminalInfo OPTIONAL,

mc			BOOLEAN,		-- shall not be set by itself

undefinedNode		BOOLEAN,

...

}

GatewayInfo ::= SEQUENCE

{

protocol		SEQUENCE OF SupportedProtocols OPTIONAL,

nonStandardData	NonStandardParameter OPTIONAL,

...

}

SupportedProtocols ::= CHOICE

{

nonStandardData	NonStandardParameter,

h310			H310Caps,

h320			H320Caps,

h321			H321Caps,

h322			H322Caps,

h323			H323Caps,

h324			H324Caps,

voice			VoiceCaps,

t120-only		T120OnlyCaps,

...,

nonStandardProtocol	NonStandardProtocol

}

H310Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H320Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H321Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H322Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H323Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

H324Caps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

VoiceCaps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

T120OnlyCaps ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix

}

NonStandardProtocol ::= SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

dataRatesSupported	SEQUENCE OF DataRate OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix,

...

}

McuInfo ::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...

}

TerminalInfo ::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...

}

GatekeeperInfo ::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

...

}

VendorIdentifier ::= SEQUENCE

{

vendor			H221NonStandard,

productId		OCTET STRING (SIZE(1..256)) OPTIONAL,	-- per vendor

versionId		OCTET STRING (SIZE(1..256)) OPTIONAL,	-- per product

...

}

H221NonStandard ::= SEQUENCE

{	t35CountryCode		INTEGER(0..255),	-- country, as per T.35

t35Extension		INTEGER(0..255),	-- assigned nationally

manufacturerCode	INTEGER(0..65535),	-- assigned nationally

...

}

NonStandardParameter ::= SEQUENCE

{

nonStandardIdentifier	NonStandardIdentifier,

data			OCTET STRING

}

NonStandardIdentifier		::=CHOICE

{

object			OBJECT IDENTIFIER,

h221NonStandard	H221NonStandard,

...

}

TransportQOS ::= CHOICE

{

endpointControlled	NULL,

gatekeeperControlled	NULL,

noControl		NULL,

...

}

Template

{

pattern			SEQUENCE OF OCTET STRING,

transportAddress	TransportAddress,

messageType		CHOICE

{

sendAddressQuery	NULL,

sendSetup		NULL,

...

}

patternType		EndpointType OPTIONAL,		-- must be present if messageType

-- is sendSetup

timeToLive		TimeSpec,

transportQoS		TransportQoS OPTIONAL,

authentication		AuthenticationMechanisms OPTIONAL,

...

}

Descriptor ::= SEQUENCE

{

zoneID			GloballyUniqueID,

gatekeeperID		GatekeeperIdentifier OPTIONAL,

templates		SEQUENCE OF Template,

...

}

ZoneRequest ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

zoneID			GloballyUniqueID OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

ZoneRequestConfirmation ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

descriptor		Descriptor,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

ZoneRequestRejection ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

rejectReason		ZoneRequestRejectReason,

zoneID			GloballyUniqueID OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

ZoneIDRequest ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

zoneIDRequestConfirmation ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

zoneIDs			SEQUENCE OF GloballyUniqueIdentifier,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

zoneIDRequestRejection ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

rejectReason		ZoneIDRequestRejectReason,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

ZoneUpdate ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

descriptors		SEQUENCE OF Descriptors OPTIONAL,

zoneIDs			SEQUENCE OF GloballyUniqueID OPTIONAL,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

AddressQuery ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

destinationInfo	SEQUENCE OF AliasAddress,

replyAddress		TransportAddress,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

AddressQueryConfirmation ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

descriptor		Descriptor,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

AddressQueryRejection ::= SEQUENCE

{

seqnum			SequenceNumber,

version			ProtocolIdentifier,

rejectReason		AddressQueryRejectReason,

tokens			SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens		SEQUENCE OF CryptoH323Token OPTIONAL,

nonStandard		NonStandardParameter OPTIONAL,

integrityCheckValue	ICV OPTIONAL,

...

}

## 42 Signaling Examples

These signaling examples are provided to illustrate basic operation. For these examples, consider the network shown in Figure 7. In these examples, assume that the administrative domains have agreements with each other, so the border elements have been provisioned with information (e.g., TCP ports) about each other.

Figure 7 - Sample Network for Signaling Examples

### 43 Exchange of Zone Information

### 44 Placing a Call

The Rapporteur’s group will provide examples.

_______________________________