Barry's CIDR/VLSM Quick Reference Guide

Ver 1.2

By Barry Raveendran Greene <bgreene@cisco.com>

The following information will help Internet Service Providers (ISPs) and network engineers remember the annotations used in today’s CIDRized Internet community.

# Classfull World - The Old Way

The IP subnetting scheme that most people are familiar with is the ‘class’ system.  In this system, IP numbers are broken into three classes: class A, class B,  class C, class D, and class E.  Each class allocated an certain number of IP host assignments.

| Class | Address or Range | # of Hosts | Status |  |
| --- | --- | --- | --- | --- |
| A | 0.0.0.0 1.0.0.0 through 126.0.0.0 127.0.0.0 |  | Reserved Available Reserved |  |
| B | 128.0.0.0 128.1.0.0 through 191.254.0.0 191.255.0.0 |  | Reserved Available Reserved |  |
| C | 192.0.0.0 192.0.1.0 through 223.255.254 223.255.255.0 | 254 | Reserved Available Reserved |  |
| D | 224.0.0.0 through 239.255.255.255 |  | Multicast |  |
| E | 240.0.0.0 through 255.255.255.254 255.255.255.255 |  | Reserved Broadcast |  |

# CIDR  -  Classless Inter-Domain Routing

In the Internet community, you will see networks expressed in CIDRized shorthand.  For example, a block of 32 networks would be expressed as:

203.127.224.0/19

When you configure this CIDR block in BGP, you use express the route as:

network 203.127.224.0 mask 255.255.224.0

Again, if this route needed to be configured in OSPF, you will need to wild-card mask:

network 203.127.224.0 0.0.31.255 area 0

Each syntax is correct. All NOC staff members must be familiar with the different expressions networks in a CIDRized Internet.

| Subnet Mask | Wild-Card Mask | Prefix |
| --- | --- | --- |
| 255.0.0.0 | 0.255.255.255 | /8 |
| 255.128.0.0 | 0.127.255.255 | /9 |
| 255.192.0.0 | 0.63.255.255 | /10 |
| 255.224.0.0 | 0.31.255.255 | /11 |
| 255.240.0.0 | 0.15.255.255 | /12 |
| 255.248.0.0 | 0.7.255.255 | /13 |
| 255.252.0.0 | 0.3.255.255 | /14 |
| 255.254.0.0 | 0.1.255.255 | /15 |
| 255.255.0.0 | 0.0.255.255 | /16 |
| 255.255.128.0 | 0.0.127.255 | /17 |
| 255.255.192.0 | 0.0.63.255 | /18 |
| 255.255.224.0 | 0.0.31.255 | /19 |
| 255.255.240.0 | 0.0.15.255 | /20 |
| 255.255.248.0 | 0.0.7.255 | /21 |
| 255.255.252.0 | 0.0.3.255 | /22 |
| 255.255.254.0 | 0.0.1.255 | /23 |
| 255.255.255.0 | 0.0.0.255 | /24 |

Subnet Mask			The subnet mask of the CIDR block.

Wild-Card Mask			Wild-Card Mask used in OSPF and packet filters

Prefix				Short hand syntax of the CIDR block.

# VLSM  -  Variable Length Subnet Mask

VLSM and CIDR are mostly one in the same. Both use subnet mask with the evenly divided octets in the old class based world.  VLSMs usually refer to CIDR addressing with prefixes /24 and above.

| # of subnets | Subnet Bits | Host Bits | Host on Subnet | Prefix | Subnet Mask | OSPF wildcard-mask |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 8 | 254 | /24 | 255.255.255.0 | 0.0.0.255 |
| 2 | 1 | 7 | 126 | /25 | 255.255.255.128 | 0.0.0.127 |
| 4 | 2 | 6 | 62 | /26 | 255.255.255.192 | 0.0.0.63 |
| 8 | 3 | 5 | 30 | /27 | 255.255.255.224 | 0.0.0.31 |
| 16 | 4 | 4 | 14 | /28 | 255.255.255.240 | 0.0.0.15 |
| 32 | 5 | 3 | 6 | /29 | 255.255.255.248 | 0.0.0.7 |
| 64 | 6 | 2 | 2 | /30 | 255.255.255.252 | 0.0.0.3 |
| 128 | 7 | 1 | 0 | /31 | 255.255.255.254 | 0.0.0.1 |

# Common Variable Subnet Breakouts

| 8 Subnets Subnet Mask = 255.255.255.224 30 (23) Host per Subnet Wildcard Mask = 0.0.0.31 8 Host numbers reserved for routers, hubs, and maintenance (sniffers) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subnet | Gateway Router | Host reserved for hub and alternate path.... | Host available for assignment (23 hosts) | Host reserved for maintenance (sniffer) |  |  |  |
| 128.244.1.0 | 128.244.1.1 | 128.244.1.2 - 4 | 128.244.1.5 - 26 | 128.244.1.27 - 30 |  |  |  |
| 128.244.1.32 | 128.244.1.33 | 128.244.1.34 - 36 | 128.244.1.37 - 58 | 128.244.1.59 - 62 |  |  |  |
| 128.244.1.64 | 128.244.1.65 | 128.244.1.66 - 68 | 128.244.1.69 - 90 | 128.244.1.91 - 94 |  |  |  |
| 128.244.1.96 | 128.244.1.97 | 128.244.1.98 - 100 | 128.244.1.101 - 122 | 128.244.1.123 - 126 |  |  |  |
| 128.244.1.128 | 128.244.1.129 | 128.244.1.130 - 132 | 128.244.1.133 - 154 | 128.244.1.155 - 158 |  |  |  |
| 128.244.1.160 | 128.244.1.161 | 128.244.1.162 - 164 | 128.244.1.165 - 186 | 128.244.1.187 - 190 |  |  |  |
| 128.244.1.192 | 128.244.1.193 | 128.244.1.194 - 196 | 128.244.1.197 - 218 | 128.244.1.219 - 222 |  |  |  |
| 128.244.1.224 | 128.244.1.225 | 128.244.1.226 - 228 | 128.244.1.229 - 250 | 128.244.1.251 - 254 |  |  |  |

| 4 Subnets Subnet Mask = 255.255.255.192 62 (55) Host per Subnet Wildcard Mask = 0.0.0.63 8 Host numbers reserved for routers, hubs, and maintenance (sniffers) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Subnet | Gateway Router | Host reserved for hub and alternate path.... | Host available for assignment (55 hosts) | Host reserved for maintenance (sniffer) |  |  |
| 128.244.1.0 | 128.244.1.1 | 128.244.1.2 - 4 | 128.244.1.5 - 58 | 128.244.1.59 - 62 |  |  |
| 128.244.1.64 | 128.244.1.65 | 128.244.1.66 - 68 | 128.244.1.69 - 122 | 128.244.1.123 - 126 |  |  |
| 128.244.1.128 | 128.244.1.129 | 128.244.1.130 - 132 | 128.244.1.133 - 186 | 128.244.1.187 - 190 |  |  |
| 128.244.1.192 | 128.244.1.193 | 128.244.1.194 - 196 | 128.244.1.197 - 250 | 128.244.1.251 - 254 |  |  |

| 2 Subnets Subnet Mask = 255.255.255.128 126 (119) Host per Subnet Wildcard Mask = 0.0.0.127 8 Host numbers reserved for routers, hubs, and maintenance (sniffers) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Subnet | Gateway Router | Host reserved for hub and alternate path.... | Host available for assignment (119 hosts) | Host reserved for maintenance (sniffer) |  |  |
| 128.244.1.0 | 128.244.1.1 | 128.244.1.2 - 4 | 128.244.1.5 - 122 | 128.244.1.123 - 126 |  |  |
| 128.244.1.128 | 128.244.1.129 | 128.244.1.130 - 132 | 128.244.1.133 - 250 | 128.244.1.251 - 254 |  |  |

# COMPLEX CIDR/VLSM NETWORKS

Many sites would like have a mix of several different subnets in their network. This allows for more efficient use of the IP address space in their organization. Yet, working through the math in insuring you are not crossing binary or octal bit boundaries can lead to problems. Attached is a reference table created by Raja Masmar Bte Raja Rom (masmar@staff.singnet.com.sg). It is a useful tool to insure your network planning.

## Binary Conversion Table

# Default Administrative Distance Between Routing Protocols

| Connected Interface | 0 |
| --- | --- |
| Static Route | 1 |
| Enhanced IGRP Summary Route | 5 |
| External BGP | 20 |
| Internal Enhanced IGRP | 90 |
| IGRP | 100 |
| OSPF | 110 |
| IS-IS | 115 |
| RIP | 120 |
| EGP | 140 |
| External Enhanced IGRP | 170 |
| Internal BGP | 200 |
| Unknown | 255 |

Available IP addresses area delegated by the Internet Assigned Numbers Authority (IANA) to regional registries. Please check with WHOIS databases at the InterNIC, RIPE, and APNIC to the current status each IP address.