| Name: | NCL1155 – Version 1.1B |  |  |  |
| --- | --- | --- | --- | --- |
| Versions: | WaveRider Communications, Inc. NCL1155 v1.1B CLI v1.2 Cryptographics v1.0 Network_Tools v1.0 Telnet v1.0 MACDriver v3.0 EXC v1.0 RIP v1.0 DHCP v1.0 FPGA Version WRM1151_0.3_FPGA_0.82 |  |  |  |
| Description: | The NCL1155 Release 1.1B will have the following fixes for problems found in the NCL11551.1A fixed: - The txrx test on a master causes and exception when no argument is supplied. - The PER command on a station will cause an exception when there are other stations transmitting. |  |  |  |
| Date: | Feb. 23, 2001, 09:09:02 |  |  |  |
| Compability: | No compatibility claimed. |  |  |  |
| SPRs Fixed: | - Throwing an exception from a low priority task causes unit to hang - DHCP Relay enabled causes problems passing TCP/IP traffic - Change an Interface properties via Telnet in Bridging Mode causing unit to crash - Ethernet Buffers not being scrubbed causing unit to stop passing traffic - Unit hangs with RIP enabled and with heavy Ethernet traffic - Adding and Deleting Stations in Bridging mode causes a broadcast storm, cutting performance in half - Slowdown problem in Bridging mode - Force Ethernet to 10BaseT - The PER command on a station will cause an exception when there are other stations transmitting. |  |  |  |
| Known Problems | - None |  |  |  |
| Loading Instructions: | With an existing NCL1155, use the following upgrade: - Use an FTP client and upload the executable image “NCL1155.exe” into the NCL1155, set to binary mode by typing bin, and transfer image by typing put ncl1155.exe and reboot. - Save. - Reboot. |  |  |  |
| CLI Changes: | None. |  |  |  |
| Filename |  | Size | Description |  |
| ncl1155.exe |  | 1,016,836 bytes | NCL1155 V1.1B Application image. |  |