# NCL1155 Upgrade Instructions

| Loading Instructions: | With an existing NCL1155, use the following upgrade: - Use an FTP client and upload the executable image “NCL1155.exe” into the NCL1155, set to binary mode by typing bin, and transfer image by typing put ncl1155.exe and reboot. - Get the executable image file “ncl1155.exe.” - From a PC that has FTP client software, open an FTP session with the NCL1155 - Login to the NCL1155 using: - nothing for a username - the NCL1155 system password for the password - Change the transfer mode to binary on the FTP client, type <bin> - Do a “put” command. TYPE <put ncl1155.exe> - The FTP client should respond with “226 Transfer Complete” - Quit your FTP client, type “bye” - Telnet into the unit, save and reset. Then check sys for version number 1.1b. |
| --- | --- |

      - FTP client – go into a dos prompt and FTP to the unit eg.  FTP xxx.xxx.xxx.xxx  ( using windows ftp client ) I have had problems with other ftp clients.

      - Press ENTER for Username and Password ( or system password )

      - Type BINARY

      - Type PUT ncl1155.exe ncl1155.exe

      - Type BYE to exit FTP

      - Connect to the radio and type WR then RE ( write and Restart )

      - Verify upgrade has worked type system version