|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 -2000 | January 1998 Original: English |

Question 14/16

STUDY GROUP 16 - CONTRIBUTION

SOURCE* :	RAPPORTEUR FOR Q.14/16 (Gary THOM)

TITLE:		PROPOSED RECOMMENDATION H.235

__________

Summary

This Document contains changes approved during SG16 Plenary in Geneva 1998 as applied to COM 16-40 (H.235)

* Contact:	Jim Toga	Tel.: +1 (503) 264-8816

Intel Corporation	Fax: +1 (503) 264-3485

Santa Clara, USA	E-mail: jtoga@ibeam.intel.com

# 4.	Definitions

authorization: The granting of permission on the basis of authenticated identification.

cryptographic algorithm: Mathematical function that computes a result from one or several input values.

### 6.6.1	Key Escrow

Although not specifically required for operation, this recommendation contains provision for entities utilizing the H.235 protocol to support key recovery technique within the signalling elements.

The ability to recover lost media encryption keys should be supported in installations where this functionality is desired or required.

Key escrow is a facility which is often referred to as a trusted third party (TTP).  This facility is for further study.

ANNEX a: H.235 ASN.1

--

Params ::= SEQUENCE {

ranInt	INTEGER OPTIONAL, -- some integer value

iv8		IV8 OPTIONAL,	-- 8 octet initialization vector

...

}

EncodedGeneralToken ::= TYPE-IDENTIFIER.&Type (ClearToken -- general usage token -- )

PwdCertToken ::= ClearToken (WITH COMPONENTS {..., timeStamp PRESENT, generalID PRESENT})

EncodedPwdCertToken ::= TYPE-IDENTIFIER.&Type (PwdCertToken)

EncodedKeySyncMaterial	::=TYPE-IDENTIFIER.&Type (KeySyncMaterial)

H235CertificateSignature	::=SEQUENCE

{

certificate			TypedCertificate,

responseRandom		RandomVal,

requesterRandom	RandomVal OPTIONAL,

signature			SIGNED { EncodedReturnSig },

…

}

ReturnSig ::= SEQUENCE {

generalId			Identifier, -- slave's alias

responseRandom 	RandomVal,

requestRandom 		RandomVal OPTIONAL,

certificate  		TypedCertificate OPTIONAL -- requested certificate

}

EncodedReturnSig ::= TYPE-IDENTIFIER.&Type (ReturnSig)